AUTOID Smartwatch-2.0M4-9.0

PRODUCT SPECIFICATION V1.0

Manufacturer: JIANGSU SEUIC TECHNOLOGY CO., LTD

# AUTOID Smartwatch-2.0M4-9.0 Performance, Physical and Environmental Parameters

| # CPU | # MTK 4-core 2.0GHz |
| --- | --- |
| # Operating system | # Android 9.0 |
| # RAM | # LPDDR3 3GB |
| # ROM | # 32GB |
| # Interface | # TYPE-C |
| # Keys | # 2 keys (power supply, custom), 1 million times life |
| # Screen | # 2.8 inch full view LCD, capacitive 3-point touch. |
|  | # The resolution is 640 × 480 and the maximum brightness is 350nit |
| # Power Supply | # Non detachable 3300mah lithium ion rechargeable battery, 500 cycles, capacity more than 80% |
| # Audio frequency | # Built in speaker, 8 Ω, 1 W * 1. Built in dual microphone |
| # Sensor | # Acceleration sensor, proximity light sensor, electronic compass, gyroscope, barometer. Support to raise your hand to light up the screen. |
| # Vibrating motor | # support |
| # Size | # 54.7*80*17mm |
| # Weight (including standard battery) | # 100 g No strap |
| # Working temperature | # -20℃ to +60℃ |
| # Storage temperature | # -30℃ to +70℃ |
| # Humidity | # 5% to 95% RH No condensation state |
| # Waterproof and dustproof industrial grade | # IP67 |
| # Drop Level | # 1.2m Marble Falls, 5 rounds on 6 sides, 30 times |
| # Electrostatic discharge (ESD) | # Air discharge of ± 15kV and direct discharge of ± 8Kv |